Trimble Business Center

Trimble Dongle License Number	:

Important Note:

Plug dongle into USB port on computer BEFORE starting TBC

If Trimble Business Center does not recognize your data collector under Device Pane, shut TBC down and restart it

For your information, this Cheat Sheet is based on the following organized file structure on my hard drive:

C:\

tmp

Jobs

2014

2015

TxxXRxxX

ArcMap

CMM

Drawings

Garmin

GCDB

MTPs and His

Notes and Plats

OPUS

TBC

TriStations

Weeklies

We also want to centralize our customized TBC export files for CMM and Garmin as well as the Arizona State Plane files, so let’s create a folder called:

C:\tmp\Jobs\TBC Custom Files\

Customizing Ribbons

Trimble Business Center is nice in that it utilizes “Ribbons” which we can customize to suit our specific Cadastral needs. When you open up TBC, you will notice that there are the following 12 tabs: File, Home, View, Data, Survey, CAD, Lines, Surface, Corridor, Photogrammetry, Point Cloud, & Support. You will also notice that under each tab, are various groups; let’s take for instance the Home tab, it has the following groups: Import/Export, Data, View, Images, Geodetic, Reports, & Print. Now under each group are various subroutines; let’s take for instance under the Home tab and in the Data group are the subroutines called Properties, Project Explorer, Selection Explore, and Select All.

Now this can get pretty confusing trying to locate the subroutine you want to execute under all of the various tabs and groups. TBC allows us to create and customize our own tabs and groups. The following process will show us how to make a tab called Cadastral with the following groups: Files, Layers, Points, Lines, Text, Survey, and Customize Appearance. This is the layout of how Cadastral Ribbon (tab) will appear; but of course you can create it however best suits your needs.

Cadastral tab

Files group

Open

Project Settings

Devise Pane

Import

Export

Save Project As Template

Save Project

Close Project

Layers group

New Layer

Select by Layer

Layer Options

View Filter Manager

Points group

Select Points

Create Point

Selection Explorer

Rename Points

Lines group

Line

Line Style Manager

Line Type Editor

Text group

Create Text

Text Style Manager

Survey group

Inverse

Measure Angle

Measure Distance

Google Earth

Customize Appearance group

New Points Spread Sheet “added in class”

Toggle Gridlines

Options

Command Pane

Creating the Cadastral Tab

Right Click in the ribbon

Customize the Ribbon…

New Tab button in the upper middle of the Options window

Towards the bottom of the Options window is New Group (Custom)

Modify Item icon to the right of New Group (Custom)

In the area of Main Tabs > New Tabs > type Cadastral

OK button

OK button

To the right side of the Options window are Up and Down Arrows. Click on the Cadastral tab and use the Up Arrow to move the Cadastral tab towards the top of the rest of the tabs.

Now we will add the Files, Layers, Points, Lines, Text, Survey, and Customize Appearance groups to our Cadastral Tab:

Creating the Groups Under the Cadastral Tab

This process will show us how to add a command to our ribbon bar. In this process, we will add the command “Rename Points” to the Home/Data tab.

Right Click anywhere in the Ribbon (which is under the tabs)

Customize Ribbon tab

Double Click Home located in the right half of Options window.

Expand Data

Drag and drop Rename Points from the left half of window into Data.

Creating State Plane Project Templates

This process will allow us to create the templates: Arizona Central, Arizona East, and Arizona West with an assortment of layers to help organize the data. We want to first create a project, establish the project settings, import Dashed 1-1 Line, create the layers, and then save it as a template.

Start Page tab located to the lower left of the Cadastral ribbon

Start a new project

Highlight US Survey Foot

OK button

“Highlighted text is the only changes that should need to be made”

Establish the Project Settings

Cadastral tab

Files group

Project Settings

Coordinate System folder in the upper left of the Project Settings window

Datum Transformation

Change… button left of center of Project Settings window

Coordinate System and Zone

Highlight US State Plane 1983

Highlight Arizona Central 0202		NAD 83 (Conus)

Next button

 Predefined geoid model:

 GEOID12B (Conus)

Geoid model quality	 Survey Quality

Finish button “If you hit the finish button you will need to go back into this menu.  Only hit the finish button if you are done making all of your changes.

Coordinate System folder

Geoid Model & Vertical Datum

Change… button

 Predefined geoid model:

 GEOID12B (Conus)

Geoid model quality	 Survey Quality

Finish button

Cadastral tab

Files group

Project Settings

Units folder

Coordinate

Coordinate

Display order:		Northing, Easting, Elevation

Formatting

Decimal Precision

Latitude / Longitude		0.12345

Coordinate:				0.123

Elevation:					0.123

Show trailing zeros:		Yes

Show trailing decimal:	Yes

Suffix

Display:					Yes

Add space:					Yes

Latitude / longitude

Format:					Degrees, minutes, seconds

Label latitude / longitude:	Yes

Show zero minutes:		Yes

Show zero seconds:		Yes

OK button “If you hit the OK button you will need to go back into this menu.  Only hit the OK button if you are done making all of your changes.”

Cadastral tab

Files group

Project Settings

Units folder

Distance

Display:					US Survey foot

Formatting

Decimal precision:		0.123

Show trailing zeros:		Yes

Show trailing decimal:	Yes

Rounding mode:			Normal

Automatic Rounding:		Yes

Suffix

Display:					Yes

Abbreviation:				ft

Add space:					Yes

OK button

Cadastral tab

Files group

Project Settings

Units folder

Angular

Unit

Display:					Degree, minute and second

Formatting

Decimal precision:		0.12345

Show trailing zeros:		Yes

Show trailing decimal:	Yes

Degrees, minute, second settings

Show zero seconds:		Yes

Show zero minutes:		Yes

DMS format				DDD  MM  SS.sss

DMS rounding mode:		None

OK button

Cadastral tab

Files group

Project Settings

Units folder

Azimuth

Units

Display:					Bearing

Formatting

Add azimuth labels:		Yes

Decimal precision	:		0.12345

Show trailing zeros:		Yes

Show trailing decimals:	Yes

Suffix

Display:					Yes

Add space:					Yes

Bearing settings

Always show North angles:	No

Always show South angles:	No

Bearing format:				N12°34'56"E

Degrees, minute, second settings

Show zero seconds:		Yes

Show zero minutes:		Yes

DMS format				DDD  MM  SS.sss

DMS rounding mode:		None

OK button

Cadastral tab

Files group

Project Settings

Units folder

Area

Unit

Display:					Square foot

Formatting

Decimal precision:		0.1

Show trailing zeros:		Yes

Show trailing decimal:	Yes

Rounding mode:			Normal

Automatic rounding:		Yes

Suffix

Display:					Yes

Abbreviation:				ft²

Add space:					Yes

Alternate

Display:					Acre

Abbreviation:				AC

Decimal precision:		0.12

OK button

Cadastral tab

Files group

Project Settings

Computations folder

Point tolerances

Survey Quality

Horizontal tolerance (Survey):	0.066 ft

Vertical tolerance (Survey):		0.164 ft

Mapping Quality

Horizontal tolerance (Mapping):	16.404

Vertical tolerance (Mapping):		32.808

Unknown Quality

Horizontal tolerance (Unknown):	32.808

Vertical tolerance (Unknown):	49.213

Merge On Import

Merge options:					By Point Tolerance x 3

OK button

Cadastral tab

Files group

Project Settings

Computations folder

GNSS Vector

Tolerance of Mean Vectors

Horizontal:				0.164 ft

Vertical:					0.262 ft

OK button

Cadastral tab

Files group

Project Settings

Computations folder

Mean Angles

Tolerances of Mean Angles

Horizontal angle:				0°00'15.00000"

Vertical angle:					0°00'15.00000"

Slope distance:				0.016 ft + 2.0 ppm

OK button

Cadastral tab

Files group

Project Settings

Computations folder

As-Staked Points

Tolerance of As-Staked Points

Horizontal tolerance:			0.083 ft

Vertical tolerance:				0.083 ft

Flag vertical out of tolerance:	No

OK button “Now you can hit the OK button”

Importing Dashed 1-1 Lines Style

Before we begin creating layers, we first want to import the Dashed 1-1 Line for both the Section Lines Unsurveyed and Subdivision layers.

Cadastral tab

Lines group

Line Style icon

Browse folder:		C:\Program Files\Trimble\Trimble Business Center\FCEdit\Linetype.ltp

Highlight:			Dashed 1-1

< button

OK

NOTE: 	Do not change the Length unit to English or the Unit reference to Ground

otherwise the imported lines do not import correctly.

Unlike TGO, TBC does not have an option to assign the dashed line to a specific ground or sheet distance. This is changed in the scale units under Properties\Line Style Scale which this process is described below under Editing Lines \ Changing a Layer’s Default Line Scale. We are unable to perform this task now until we actually have lines already drawn up in the Section Lines Unsurveyed and Subdivision layers.

Creating Layers

Cadastral tab

Layer group

New Layer

Layer name:	Accessories

Color:			Cyan

Apply button

Layer name:	Calculated

Color:			Yellow

Apply button

Layer name:	Check

Color:			Orange

Apply button

Layer name:	Control

Color:			Magenta

Apply button

Layer name:	Gas Lines

Color:			Yellow

Line Style:		Solid

Apply button

Layer name:	GCDB

Color:			Medium Green

Apply button

Layer name:	Local

Color:			Grey

Apply button

Layer name:	Points

Color:			Green

Apply button

Layer name:	Power Lines

Color:			Red

Line Style:		Solid

Apply button

Layer name:	Roads

Color:			Brown

Line Style:		Solid

Apply button

Layer name:	Search Areas

Color:			Pink

Apply button

Layer name:	Section Lines

Color:			Red

Line Style:		Solid

Apply button

Layer name:	Section Lines Unsurveyed

Color:			Dark Grey

Line Style:		Dashed 1-1

Apply button

Layer name:	Subdivision

Color:			Green

Line Style:		Dashed 1-1

Apply button

Layer name:	Section Numbers

Color:			White

Apply button

Layer name:	Topo

Color:			Dark Grey

Apply button

Layer name:	Township

Color:			Blue

Line Style:		Solid

Apply button

Layer name:	Trash

Color:			Dark Grey

Apply button

Layer name:	True

Color:			White

Apply button

Layer name:	True Not Set

Color:			Yellow

Apply button

Layer name:	Water Lines

Color:			Blue

Line Style:		Solid

Apply button

Saving Project as Template Arizona Central 0202

Now that we have established our project settings with a state plane zone of Arizona Central 0202, imported Dashed 1-1 Line, and created our various layers, all of the tedious work is done and we will now save this as a template entitled Arizona Central 0202

Cadastral tab

Files group

Save Project as Template icon

Name:		Arizona Central 0202

Save button

Creating Template Arizona East 0201

All of the tedious work has been done! We will take the now saved Arizona Central 0202 template and change the state plane zone to Arizona East 0201 and save it as template Arizona East 0201

Cadastral tab

File group

Project Settings icons

US Survey Foot tab in the lower right of your screen

Coordinate System folder in the upper left of the Project Settings window

Datum Transformation

Change… button left of center of Project Settings window

( Coordinate System and Zone

Highlight US State Plane 1983

Highlight Arizona East 0201		NAD 83 (Conus)

Next button

( Predefined geoid model:

b GEOID12B

Geoid model quality	b Survey Quality

Finish button

OK button

Files tab

Save Project as Template icon

Name:		Arizona East 0201

Save button

Creating Template Arizona West 0203

US Survey Foot tab in the lower right of your screen

Coordinate System folder in the upper left of the Project Settings window

Datum Transformation

Change… button left of center of Project Settings window

( Coordinate System and Zone

Highlight US State Plane 1983

Highlight Arizona West 0203		NAD 83 (Conus)

Next button

( Predefined geoid model:

b GEOID12

Geoid model quality	b Survey Quality

Finish button

OK button

Files tab

Save Project as Template icon

Name:		Arizona East 0201

Save button

Importing CMM into Trimble Business Center

This process will take us from creating a custom export file in CMM to creating a custom import file in Trimble Business Center.

NOTE:	If your points in TBC are on various layers, it is HIGHLY advisable to ONLY import those points that you calculated in CMM and not to export points that are already in TBC, otherwise, all of your points in TBC will be automatically moved from your various layers to the Points layer. The points that you already have in TBC can be thinned out of the *.cor and *.lev files in CMM:

Project_Files

View Project Reports w/WordPad

In CMM, Creating Custom *.csv Export File

This process creates a *.csv file which is a Microsoft Excel Comma Separated Values file. This process is not a onetime set up but will have to be performed EVERYTIME when exporting from CMM to Trimble Business Center

File

Export

User Defined Export

Input File:		C:\tmp\Jobs\year\TxxNRxxE\CMM\TxxNRxxE.def

Output File:	C:\tmp\Jobs\year\TxxNRxxE\CMM\TxxNRxxE.csv

1: 	b Name

2: 	b Northing

3: 	b Easting

4: 	b Elevation

5: 	b blank

6: 	b blank

7: 	b blank

Geo Target Format:	dddmm’ss.s” N/W

Output File Delimiter:	b Comma

Precision

Grid:		4

Geodetic:	5

Elevation:	3

In TBC, Creating Custom *.csv Import File

This process will set up a custom import file format for the CMM.csv file and this process will only have to be done once.

Cadastral tab

Files group

Import icon

Import Format Editor icon near the top middle of window.

New button

Definition name:	CMM.csv

Next button

Description:	Name, Northing, Easting, Elevation

( Delimited

… Browse button 																							C:\tmp\Jobs\year\TxxNRxxE\CMM\TxxNRxxE.csv

Next

Delimiter:					Comma

Store Points as:			Points

Default File Extension:	.csv

		Show editor on import

Coordinate Quality:		Survey

Next

Fields button

Point ID

Fields button

Northing

US Survey Foot

 Apply to all

Fields button

Easting

US Survey Foot

 Apply to all

Fields button

Elevation

US Survey Foot

 Apply to all

Finish button

In TBC, Importing CMM.csv File

This process will import our CMM.csv file (created in CMM) into TBC.

Cadastral tab

Files group

Import icon

Import Format Editor icon near the top middle of window.

… Browse button located in the middle and towards the right of the window.

C:\tmp\Jobs\year\TxxNRxxE\CMM\TxxNRxxE.csv

Import button

Highlight CMM.csv

Exporting Comma Delimited File from TBC to CMM

The general overall process of exporting from TBC to CMM and from CMM back into TBC is as follows:

- In TBC, export *.nez file

- In CMM, import *.nez file and convert to *.lsa file

- Perform calculations in CMM

- Thin out *.cor and *.lev files to ONLY contain newly calculated positions.

- In CMM, export *.csv file

- In TGO, import *.csv file

This process will allow us to create a comma delimited file in the format of:

name, northing, easting, elevation. CMM will then use this file to create a township.cor and a township.lev file.

In TBC, Creating the Custom CMM.nez Export File Editor

Cadastral tab

Files group

Export icon

Custom tab

Export Format Editor icon near the upper center of window.

New button

Type CMM.nez under the Definition Name header.

Next button

Format definition description: name, northing, easting, elevation

Uncheck box in front of Include header

 Delimited

Default file extension:	  .nez

Decimal separator:  .

Encoding:  ASCII

Next button

Available Fields:	expand General

Double Click Point ID

Double Click Northing

Double Click Easting

Double Click Elevation

Click on Northing tab

Units   US Survey Foot

Click on Easting tab

Units   US Survey Foot

Click on Elevation tab

Units   US Survey Foot

Finish button

In TBC, Exporting the CMM.nez File into Your CMM Folder

With this process, we will select our points from the True layer and export them into CMM folder.

Cadastral tab

Points group

Select Points icon

General tab

Layer b True

OK button

Cadastral tab

Files group

Export icon

Custom tab

Single Click CMM.nez

File Name:		C:\tmp\Jobs\year\TxxxNRxxE\CMM\TxxNRxxE.nez

Click the ... browse button and navigate to your CMM folder.

Start your CMM program and open up your job.

File

Import

Convert NEZ to LSA

Open button

Navigate to and highlight your C:\tmp\Jobs\year\TxxxNRxxE\CMM\TxxNRxxE.nez

OK button

Save button

Project Definition window opens up. Select your correct State Plane Zone, project elevation, ect.

Exit button

Exit button

NOTE: Don’t panic! It appears as if your points were not imported, but they did; we just need to refresh the your view in CMM

File

Refresh F3

Zoom

Zoom Extents

GCDB – WinGMM

Before we start a project, we will take a small detour from TBC and use WinGMM to convert our GCDB data from NAD27 to NAD83. For Arizona GCDB data, go to:

http://www.blm.gov/az/st/en/prog/cad/downloads.html

GCDB folder

Survey-Data-Gila-and-Salt-River-Meridian folder

Quadrant folder

TxxXRxxX.zip

Store zip file in C:\tmp\Jobs\year\TxxXRxxX\GCDB

In WinGMM, we will unzip the folder and then convert from NAD27 to NAD83.

Open up WinGMM

File

Zip Maintenance

Extract files from a ZIP archive

C:\tmp\Jobs\year\TxxXRxxX\GCDB

Highlight TxxXRxxX.zip

 Set target folder as current WinGMM working folder

Convert LF to CRLF

OK button

OK button

File

Open

TxxXRxxX.def

Open button

Command

Datum Transformation NAD27<>NAD83 (GCONW)

Enter

Enter

Enter

Enter

After you have created a CMM project for this township in your CMM folder, navigate to C:\tmp\Jobs\year\TxxXRxxX\GCDB and copy and paste the TxxXRxxX.cor and TxxXRxxX.lev files into C:\tmp\Jobs\year\TxxXRxxX\CMM.

Now follow the steps under the heading Importing CMM into Trimble Business Center

Starting a New Project

This process will show us how to start a new project using one of our newly created Arizona State Plane templates and save it in our desired folder location. But first, make sure you have all of your folders created for your particular project as illustrated on page 1.

Start Trimble Business Center and click on the Start Page tab to the lower left of the ribbon.

Start a new project

Arizona zone 020x

OK button

Cadastral tab

Files group

Save

HIGHLY IMPORTANT – Project File Locations

When starting a new job, we want to all of our import files and export files to be in specific locations.

Cadastral tab

Custom Appearance group

Options icon

General folder

File Locations also Project Management

Project management folder:	C:\tmp\year\TxxXRxxX\TBC\TxxXRxxX

Export folder:					C:\tmp\year\TxxXRxxX\CMM

Download & import folder:	C:\tmp\year\TxxXRxxX\TBC\TxxXRxxX\Import

 Copy imported files to import folder

Folder for intermediate report: C:\tmp\year\TxxXRxxX\TBC\TxxXRxxX\Report

Block definition file folder:	C:\ProgramData\Trimble\Block Definitions\

Template folder:				C:\tmp\Jobs\TBC Custom Files\

Format definition folder:		C:\tmp\Jobs\TBC Custom Files\

OK button

NOTE: When we made our Arizona State Plane templates, they may have been created and stored in C:\tmp so we will want to cut and paste them into C:\tmp\Jobs.

Renaming Points

This process will add the prefix “G” to our GCDB points.

Cadastral tab

Points group

Rename Points icon

( Add prefix

G

OK button

Project Location and Height

The data collector will not function properly out in the field unless we first have the project in TBC set up with a project location and height.

Cadastral tab

File group

Project Settings

Coordinate System folder

Local Site

Local Site Settings button

Coordinate type:	Grid

Northing:			Click in the Northing field and then click on G400400,

this will put both the northing in the Northing field and

the easting in the Easting field.

Elevation:			Enter average project elevation

Un Check Use ground coordinates

Ground scale factor:	1.0000000000

False northing offset:	0.000 ft

False easting offset:	0.000 ft

OK button

Turning Layers ON & OFF

Home

View

View Filter Manager icon

Layers

Point Labels

To toggle Point Labels ON & OFF:

Home

View

View Filter Manager icon

Point tab

Show Point Labels

Vector Lines

To toggle RTK Vector Lines ON & OFF:

Home

View

View Filter Manager icon

Raw Data

RTK Vector

Importing From TSC2

Connect TSC2 data collector to the computer.

Cadastral tab

Files group

Device Pane button

Device Pane window opens. Expand the Survey Controller by clicking on the +

Double Click your job (or you can drag and drop your job from the Device Pane and into the Plan View of your job.

NOTE: If your controller does not show up in the Device Pane window, shut down and restart Trimble Business Center.

Alternate Method of Importing From TSC2

For some strange reason I was having problems with TBC connecting to the TSC2. Here is an alternate method of dragging and dropping from Windows Explorer to TBC

- Open up your job in TBC.

- Connect TSC2 to computer.

- Open Windows Explorer and reduce the Windows Explorer window to a small window.

- + Computer

- + TSC2

- Click on Trimble Data

- Drag and Drop field file from Windows Explorer into our opened job in TBC.

Every time you download, all of your data will be been imported into the Points layer. This next process will transfer our points from the Points layer to our other various layers such as the True and Topo layers.

Cadastral tab

Points group

Select Points button

General tab

Layer:	Points

OK button

Now that our points have been selected, we will move these points to their appropriate layers; in this example we will move certain points to the true layer.

Cadastral tab

Points group

Selection Explorer button

Hold down the Ctrl button and Left Click on all of the points you want moved to the True layer

Right Click somewhere in the Plan View of TBC

Properties

Layer:	True

Close button

Exporting to TSC2

Connect TSC2 data collector to the computer.

Turn on all layers you want exported and turn off all layers you do not want exported.

Select all points of the “active” layers using Ctrl A

Cadastral tab

Files group

Device Pane button

Export icon

Highlight Survey Controller job file exporter

Under File Name (Survey Controller) type:   filename.job

Export button

Creating Lines

This process will allow us to put in various lines such as Township, Section, Subdivisional, mineral survey, wilderness boundaries, and various other lines into TBC thus make viewing points much easier. In the following examples, we will be making Dashed 1-1 lines in the Subdivision layer so that you can see how to change the size (scale) of the dashed lines.

These lines are also necessary when making maps in ArcMap.

Importing Line Style

The solid line is already a default line in TBC, however, we need to import a Dashed 1-1 line to be used in both the Section Lines Unsurveyed  and Subdivision layers.

Line tab

Style group

Line Style icon

Length unit:		# English

Unit reference:	# Ground

Browse folder:		C:\Program Files\Trimble\Trimble Business Center\FCEdit\Linetype.ltp

Highlight:			Dashed 1-1

< button

OK

Unlike TGO, TBC does not have an option to assign the dashed line to a specific ground distance or sheet distance. This is changed in the scale units under Properties\Line Style Scale which this process is described below under Changing a Layer’s Default Line Scale.

Creating Layer with Line Style

CAD tab

Drafting Group

New Layer

Layer Name:	Subdivision

Color:			Green

Line Style:		Dashed 1-1

OK

Creating Polyline in Subdivision Layer

Lines tab

Create group

Name:		Subdivision

Layer:		Subdivision

Elevation:	(I leave blank)

Automatically close by connection ends:		Never

# Specify Individual Points

Click in Next point: field

Next, I would start drawing the lines in between the desired points. While the line is green, the line is not dashed. If I

Creating Lines in a Specific Layer

We want to create lines between points only, so we want the end points of the lines to “snap” to ONLY points. At the bottom (and just right of center) of the TBC window is a small icon called SNAP. Left-Click on SNAP and make sure that ONLY Points is checked.

For this illustration, we will create lines in the Section Line layer.

Lines tab

Create group

Create Polyline

Name:		Section Lines

Layer:		Section Lines

Elevation:	(I leave blank)

Automatically close by connection ends:		Never

Specify Individual Points

Click in Next point: field

Begin clicking on each of the points you want to draw lines in between.

To begin a new polyline, click New button at the bottom of the Create Polyline window.

Creating Section Numbers

Cadastral tab

Text group

Text Style

Choose New

Style Name Section Numbers

Font:			Times New Roman “you may have to check or uncheck Stroke fonts”

Justification:	Center Middle

Font Style:		Bold

Height:			600

( Ground Units

OK button

Cadastral tab

Text group

Text

Layer:					Section Numbers

Style:					Section Numbers

Justification:			Middle Center

Height:					600

Rotation:				90°00'00.00000"

Leader type:			No leader

Text:					type in Section #

Text insertion point:	left click in the township where you want the Section #

Apply button

Helpful Hint:	What I like to do is type in a capital X in the Text field, right click in the Text insertion point field and left click on Free Snap. This will give you a big cross hair so that you can align the section numbers in the center of the section. Once you have entered an X in all of the sections, then you can right click on each X, click Properties, and change the X to its appropriate section number.

Editing Lines

Changing Polyline into Individual Line Segments

This process will break the “Section Lines Unsurveyed” polyline (which is a grey Dashed 1:1 polyline) into individual ½ mile line segments so that we can later convert them to Township, Section, or Subdivision lines. We want to break the polyline at the section and quarter corner points.

Left Click and then Right Click on a polyline.

Break Line

Distance Alone:	Right Click

Station at Point

Left Click on a True point

Break button

Keep repeating this process until the polyline is broken into line segments.

Now we will change the line segment from Section Lines Unsurveyed to Section Lines.

Left Click the line you want to change from Section Lines Unsurveyed to Section Lines.

Properties

Layer:	Section Lines

Close button

Changing a Layer’s Default Line Scale

This process will change the “Section Lines Unsurveyed” Dashed 1:1 default line scale of 1 to 150. Once all of the polylines have been created from point to point in the “Section Lines Unsureyed” Layer, we will select all of the lines in the layer.

Select by Layers icon

 Section Lines Unsurveyed

Close button

Now that all of the polylines are selected:

Right Click

Properties

Line style scale:	150

Moving Points C* Points to Calculated Layer

We want to open up the list of all points:

Home tab

Data

Project Explorer

A list of ALL of the project points are in the list.

Left Click the top C* point.

Hold down the Shift button and Left Click bottom C* point.

Now that the C* points are highlighted in yellow, we will put them into the Calculated Layer.

Home tab

Data

Properties

The Properties window opens up.

Point Information

Layer

Calculated

Viewing Points in a Certain Layer

If we want to view the points in the Calculated Layer:

Data tab

Select

Select by Layer

Select by Layer window opens

Calculated

Now all of the points in the Calculated layer are selected.

Data

Select

Selection Explorer

Creating User Defined Shortcuts

This process will show you how to create user defined shortcuts with the Function, SHIFT, CTRL, and or ALT keys. In this example we will assign the F3 key to select points and the F7 key to compute inverses.

Support

Customize

Define Command Shortcuts

Command:		Select Points

Shortcut:		F3

Support

Customize

Define Command Shortcuts

Command:		Inverse

Shortcut:		F7

OPUS

Note:	-Static data only; the antenna must remain unmoved throughout the observing session.

-15-minutes of data or more, up to 48-hours, but not crossing UTC midnight more than once.

-Files under 2 hours, processed as rapid-static, must include the P2 and either P1 or C1 observables.

-GLONASS or Galileo observables may be included; though only the GPS are used.

-Any elevation cut-off or mask angle; though only satellites more than 10° above the horizon are used.

-Recording (epoch) rates of 1, 2, 3, 5, 10, 15, or 30 seconds; though all are decimated to 30 seconds.  Trimble RTX uses 10 second data.

TSC2 data collector

Configuration

Survey Styles

R8-4 INFILL (or whatever you named your R8 style)

Base options

Survey Type:		RTK & infill

Broadcast format:	CMRx (or CMR+ if not an R8 Generation 4)

Station index:		your choice

Logging device:	Receiver

Logging interval:	10s

Elevation mask:	10s

Type:				R8-4 Internal (or whatever your antenna type is)

Measured to:		Bottom of antenna mount

Antenna height:	?

Serial number:		?

Use L2e			Yes

GPS L2C			

GLONASS		

GPL L5			

Galileo				

Accept (or Enter)

Store button in lower left of screen

Esc button

Trimble RTX

Trimble RTX can be accessed by importing a Trimble Static file into a TBC project.  You can also access the service by going to www.TrimbleRTX.com.

Trimble RTX Supports GPS, GLONASS, QZSS and BeiDou.

Observation files must meet the following requirements:

- Data formats accepted include Trimble proprietary data formats (e.g. DAT, T01, T02, Quark) and the standard RINEX 2 and RINEX 3 data formats

- For optimal processing results, it is recommended to provide at least 60 minutes of observations.

- Data files cannot exceed 24 hours in length

- Data files must be static only

- Data files must contain dual frequency pseudorange and carrier phase observations (L1 and L2)

- Data must have been collected after 14 May 2011

- BeiDou data is included since 04 June 2014

- If your observation data consists of several files, please compress them to a ZIP archive and upload the zipped file. All files in the ZIP archive must belong to the same station.

Creating an R8 Device Connection in TBC

Cadastral tab

Files group

Device Pane

Computer b icon in the upper left of window

Survey Devices

b to the right of Task near the upper right of window

Options

Edit Devices

New button

GPS Receiver (R/SPS/5000/Series)

OK button

Port:	COM1

Next button

Enter name for new device:	R8 (or whatever you wish)

Next button

Maximum baud rate:	38400

Parity:					None

Max retries:			5

Timeout (secs):		10

Next button

Display summary upon completion

Finish button

Downloading Static Data from R8 to TSC3 to TBC

TSC3 Data Collector

This process will allow us to transfer the Static file (which is located in the BASE Receiver) and transfer it to the TSC3 Data Collector. So we first need to turn on the BASE Receiver and the TSC3 data collector. Then we need to connect the data collector to the receiver using Bluetooth.

Turn on R8 Receiver

Turn on TSC3 Data Collector

In the TSC3 Data Collector

General Survey

Instrument

GNSS Functions

Base Mode

“Connecting to GNSS Base via Bluetooth:

Trimble R8 123456789

Import files

New Screen opens

Import from receiver

Highlight filename “Make sure a check mark is next to the file name you want to import”  Multiple files can be imported

Import

Start

IMPORTANT NOTE:	A folder (named with your job currently opened) with a subfolder, named Other Files, was automatically created in your currently opened job in the TSC3 data collector.  If no job is open file will be downloaded into your main folder.

Downloading Static file from TSC3 Data Collector to TBC

Open up your job in Trimble Business Center.

We first need to make sure that your file will be automatically transferred to the location of your desired folder.

Cadastral tab

Customize group

Options

Project Management

Download and Import Folder:

C:\tmp\Jobs\year\TxxXRxxX\TBC\TxxXRxxX\Data Files\Trimble Files\

OK button

Cadastral tab

Files group

Device Pane

Expand Trimble Access

Expand BLM – Projects

Expand Current Job folder

Expand Other Files

Double Click job file.

Open up Windows Explorer and Cut & Paste from:

C:\tmp\Jobs\year\TxxXRxxX\TBC\TxxXRxxX\Data Files\Trimble Files\

to:

C:\tmp\Jobs\year\TxxXRxxX\OPUS

Rename the job file that you just transferred to the OPUS folder to the name of the point you collected the static information on, i.e. BASE1

Inserting OPUS Point into TBC

Once we get our OPUS solution, we can insert the point by copying and pasting from the OPUS solution report to TBC.

Cadastral tab

Points group

Create Point

Point ID:			base name

Layer:				Control

Coordinate type:	Global

Latitude:			latitude from OPUS report N (put “N” immediately after latitude)

Longitude:			longitude from OPUS report W (put “W” immediately after longitude)

Height:				EL height M (put “M” immediately after height for meters)

Elevation:			ortho height M (put “M” immediately after elevation for meters)

Status:				Enabled

On the right side of the “Create Point” pane, change each of the three “?” to Control Quality			Add button

Send Static Data to Trimble for Post-Processing

Unlike TGO where we would download the static session with a file format of *.t02, convert it to a *.DAT, convert it to a RINEX file, and then finally submit it to NGS; with Trimble RTX Post Processing website, all we have to do is upload the *.t02  directly to Trimble RTX website. Trimble will e-mail you the result in 2 different file formats: *.xml and *.pdf. Take the *.xml file and drag and drop it into your TBC job… pretty sweet!

Registering on Trimble Website

The first thing we need to do is register on Trimble’s website: http://www.trimblertx.com/Register.aspx

I am under the impression that we can process our data free for 1 year. I’m not sure if we will have to start paying or just reregister free for another year.

Submitting Static Session to Trimble TRX

Go to the website http://www.trimblertx.com/UploadForm.aspx to submit your static session.

We will be uploading the *.t02 file which

Coordinate System:			NAD83-2011

Tectonic Plate:				North America

Select a file to upload:			C:\tmp\Jobs\year\township\OPUS\

Provide your email address: 	yourname@blm.gov

Process button

Uploading Trimble Solution into TBC

Open up your e-mail, and download both the *.xml and *.pdf Trimble solutions to your hard drive:

C:\tmp\Jobs\year\township\OPUS

At this point, I would highly suggest opening up Windows Explorer and renaming the Trimble generated file to an easy to remember file name such as BASE1.

Open up Windows Explorer, navigate to your *.xml file in C:\tmp\Jobs\year\township\OPUS

and reduce the window of Windows Explorer to a smaller window, say 3” x 3” for example.

Open up your job in Trimble Business Center. Now click on Windows Explorer so that the small window of Windows Explorer is “on top of” your TBC job. Drag and drop your *.xml file into your TBC job. Your Trimble RTX Post Processed static session  is now in your job data base.

Exporting to Garmin

This process will allow us to create an export file format to go from TBC to TrackMaker and then to our Garmin. However, we will have to do a quick edit of the garmin.txt file by adding “WP,D,” to the beginning of each line in Microsoft Word.

Creating Export Garmin Format File

Cadastral tab

File group

Export b

Export Format Editor

New button

Definition Name:	Garmin

Format definition description:

 Include header

Record type:				 Delimited

Delimiter 				Comma

Text Qualifier				None

Default file extension:		.txt

Decimal separator:		.

Encoding:					UNICODE

Next button

Data type:	 Point

+ General

Double Click Point ID

Double Click Description 1

Double Click Description 2

Double Click Latitude

Units:	Decimal degrees 

 Apply to all

Double Click Longitude

Units:	Decimal degrees 

 Apply to all

Finish button

Exporting Selected TBC Points into TrackMaker Format File

In TBC, select the points you want to export.

Right mouse click one of the points and choose properties

In the Description 1 field type WP

In the Description 2 field type D

Then close the command

Cadastral tab

File group

Export

Custom tab

Garmin

File Name:		TxxXRxxX.txt

Browse button		C:\tmp\Jobs\year\TxxXRxxX\Garmin

 Close command after export

Export button

Editing Garmin.txt File in Microsoft Word

In this process, we need to add “Datum,WGS84,WGS84,0,0,0,0,0”  to the top of the file

Open up your file in C:\tmp\Jobs\year\TxxXRxxX\Garmin\garmin.txt in Microsoft Word.

You will notice that below is an example of what your file looks like:

100200,36.40409,-110.77589

100240,36.41133,-110.77589

100300,36.41858,-110.77589

100340,36.42583,-110.77589

100400,36.43308,-110.77589

100440,36.44033,-110.77589

100500,36.44758,-110.77589

100540,36.45483,-110.77589

100600,36.46208,-110.77589

100640,36.46933,-110.77589

At the top of the file, add:

Datum,WGS84,WGS84,0,0,0,0,0

Your file should now look similar to this:

Datum,WGS84,WGS84,0,0,0,0,0

100200,36.40409,-110.77589

100240,36.41133,-110.77589

100300,36.41858,-110.77589

100340,36.42583,-110.77589

100400,36.43308,-110.77589

100440,36.44033,-110.77589

100500,36.44758,-110.77589

100540,36.45483,-110.77589

100600,36.46208,-110.77589

100640,36.46933,-110.77589

I recommend saving this to a text file that you can copy and past from.

Exporting a Shape File for ArcMap

This process will allow us to export our township lines, section lines, subdivision lines, & mineral survey lines as well as point names into a geo-referenced shape file that will be used in ArcMap.

NOTE:	Unfortunately, when creating a shape file, TBC combines all of the lines from the Township Lines, Section Lines, Subdivision Lines layers into only one line shape file and NOT individual lines for each layer. So we will have to export one line layer at a time, and change the name of Line.dbf, Line.prj, Line.shp, & Line.shx to Township.dbf, Township.prj, Township.shp, & Township.shx in Windows Explorer. While this, at first may seem like a tedious process, the resulting outcome is well worth it.

So turn on only the line layer (such as Township Lines) you want to be visible in ArcMap and hit CTRL-A to select all of the lines that are visible in that particular layer.

Cadastral tab

Files group

Export icon

GIS tab

Shapefile exporter

File Name:		C:\tmp\year\TxxXRxxX\TBC\ArcMap\TxxXRxxX.shp

 Close command after export

Settings

Distance Units:		Meters

Output Horizontal:	Grid: North, East

Output Vertical:		Elevation

Name:					Yes

Layer:					Yes

Feature Codes:		No

Global Coordinates:	No

Local Coordinates:	No

Grid Coordinates:		Yes

Horizontal precision:	No

Vertical precision:		No

Date recorded:			No

Time recorded:		No

Export button

In Window Explore, navigate to C:\tmp\year\TxxXRxxX\TBC\ArcMap\ and change Line.dbf, Line.prj, Line.shp, & Line.shx to that corresponding layer such as Township.dbf, Township.prj, Township.shp, & Township.shx. Repeat this process for each layer that has lines in it that you want to be visible in ArcMap.

Exporting Road Intersections from ArcMap to TBC

This process utilizes the point and click Identify button in ArcMap to calculate the latitude and longitude of road intersections on the topographical map and then copy and paste the latitude and longitude into a Word document. Once all of the road intersections have been calculated and put into a Word document, we will remove all of the minute and second symbols and replace the degree symbol with a decimal point and then add a generic elevation to each point. After the Word document has been changed into a *.txt format file, we will then import it into TBC.

Copy and Paste from ArcMap to MicroSoft Word

I will not go into much detail about ArcMap because that is an entirely different beast to address. ArcMap

Zoom in close to the road intersection.

Click on the blue Identify button.

In the middle of the new window which just opened up, the point is identified with a meters value. To the right of the meters, click on a very small pull down arrow and choose Latitude and Longitude Degrees, Minutes and Seconds. Highlight the Latitude and Longitude by dragging your cursor and hit CTRL C.

MicroSoft Word

Open a new document in Word and on the very first line type 01 and hit the Tab button. This will now start to automatically start number our road interesections.

CTRL V pastes

Coordinate Transformation

This process will shift our coordinates from one location (at a known Northing and Easting) to a different location (at a new Northing and Easting). Let’s say that we are looking for 500600 that was originally surveyed in 1883 and we were unable to locate that monument. In the 1883 notes, the original surveyor called top of ridge North 13.20 chains from 500600. So we perform Continuous Topo along the top of the ridge calling the first point RDG1 and we end the Continuous Topo survey with RDG89 . We then transfer those points from TBC to CMM and calculate only one point from RDG1 due South 13.20 chains and call it XRDG1. Next, export XRDG1 into your TBC job and perform the following task in TBC:

Select all of the points you want to move by dragging a box around them.

Edit tab

Transform group

Transform Survey Points icon

Calculation Method	 Helmet

 Delta Northing

 Delta Easting

 Delta Elevation

 Fix Scale 1.00

Place cursor in From point / to point field

Click on RDG1 and then click on XRDG1

Uncheck box  H. Residual     V. Residual

Apply button.

Caution:	This is not a perfect coordinate transformation. In CMM I calculated XRDG1 South 13.20 chains from RDG1 and XRDG89 South 13.20 chains from RDG89, then Transform Survey Points from RDG1 to XRDG1, and then inversed in between RDG1 to XRDG1 and the distance was 0.00 ft. However, when I inversed in between RDG89 to XRDG89, the distance was 0.06 ft. So this is not a perfect transformation possibly due to XRDG1 and XRDG89 were calculated on a geodetic bearing and the transformation was done on a plane.

Trouble Shooting

If Trimble Business Center does not recognize your data collector under Device Pane

- unplug your data collector and plug it back in

- shut down and restart TBC.

- restart your PC

Vector Lines – To turn on or off the RTK vector lines,

Manager in the Layer group

b  My Filter

+ Raw Data

Uncheck RTK Vector

Miscellaneous

Original Trimble Business Center line file in located in:

C:\Program Files\Trimble\Trimble Business Center\FCEdit\Linetype.ltp

Customized line file is located in:

C:\Program Files (x86)\Trimble\Feature Definition Manager\PEditor\Linetype

Turning Grid Lines Off

This process toggles on and off the grid lines in TBC.

Home tab also under your custom tab.

View

Toggle Gridlines icon

Opus Supplemental.

If the RTK&Infill Survey Style was used in the field, the raw GPS file at the Base can be sent to OPUS to bring accuracy to the survey (required for State Plane, UTM, etc). The Infill option also allows Rover points to be observed outside the connection from the Base station (or VRS cell connection).

TBC has now simplified the entire OPUS submittal process. Once the raw data file (*.dat, *.T01, *.T02) has been downloaded, go to the File/TCC pull down menu in TBC, select Trimble

You may also go to this link directly.

https://www.myconnectedsite.com/site/surveyadministration/TrimbleAccessTools/Data%20Processing

Access Services/Survey Tools and then the Data Processing option. Then complete the prompts: browse for the file; select the appropriate service (OPUS Static); confirm the point ID, antenna type, antenna measurement type and antenna height; then provide the appropriate email address for the return and press Send.

The results will be emailed to the address provided, usually within 30 minutes. Open the email and review the results. Be sure the % of data used and the RMS values are within the project tolerance, then save the XML attachment to the TBC Project directory.

Open the Project, select Import. Select the Project directory and the XML file. Using the OPUS/XML import in TBC 2.30+ will only allow the Local (most often in the US, NAD83) position. If ITRF is required, it must be keyed in. Click Import and the following screen will appear:

After the OPUS return has been imported, the Base point is now accurate, HOWEVER IT WILL BE NECESSARY TO USE THE PROJECT EXPLORER TO RENAME THE OPUS POSITION THE SAME AS THE BASE POINT ID USED IN THE FIELD AND TO CREATE A WGS84 POSITION AT THAT POINT WITH THE STATUS OF CONTROL.

Open Project Explorer and browse the OPUS point to show Properties and rename the point to match the Base point ID in the field. Right click on the point in Project Explorer and use the Add Coordinate command, choose Global for the Coordinate Type and set the status to Control. The Global classification is required if the points is to be used in any sort of Site Calibration.

Once this is completed, import the Job/DC file and if necessary merge the multiple Base points.

NOTE: with some Trimble receivers the raw data file (DAT, T01, & T02) for the Base which contains the AUTONOMOUS point position will come into TBC as a CONTROL point. When TBC sees the position conflict between the existing OPUS position and the autonomous position it will open the Merge on Import screen and if checked it will import the data file BUT SINCE IT IS CONTROL AND IT IS THE LATEST IMPORT, THE AUTONOMOUS POSITION WILL BE HELD – THAT IS TO SAY, THE SURVEY IS NOW BASED ON THE AUTONOMOUS RATHER THAN THE OPUS POSITION. TO FIX THIS PROBLEM, GO TO THE POINT MANAGER AND RECLASSIFY THE DATA SOURCE (DAT, T01 OR T02) QUALITY TO “UNKNOWN” AND THEN RECOMPUTE THE PROJECT, OR IMPORT THE RAW DATA FILE BEFORE THE OPUS VALUES.

TBC Points and Classifications

SYSTEM DIVIDENDS, THE MOST EXPERIENCED GPS TRAINERS IN AMERICA 45

POINT MANAGEMENT:

DUPLICATE POINTS, MULTIPLE OBSERVATIONS TO A POINT, MUTILPLE DOWNLOADS OF SAME DATA COLLECTOR FILES:

WHILE CONSIDERING THE FOLLOWING ISSUES, BE SURE TO DISTINGUISH BETWEEN OBSERVATIONS AND POSITIONS DERIVED FROM THOSE OBSERVATIONS – ALSO BE SURE TO CONSULT THE LATEST TRIMBLE MANUALS AND README FILES.

Accidental and deliberate occurrences of station point names have long been an issue with field files and office database files. The Access Controller internally handles the problem using data types (post processed data or RTK), classifications (the highest in class determines the point used).

TBC has several other issues however, not the least of which is downloading an ongoing RTK file multiple times, potentially creating many copies of the same point/position.

Basically the highest classification of a particular point is the default TBC display (Control, Survey, Mapping, and Unknown) which have different icons in the point properties box. This behavior is essentially the same as in the Access Controller and fine as far as it goes and accounts for observations from the field deferring (at least initially) to Control positions imported from an ASCII file or NGS data sheet for example. However as the points may come from a variety of sources, undergo several different levels of processing, adjustment, etc., TBC must have additional rules governing the current position for any given point.

1. If a station has a point quality of Control, that position trumps any other position, that is to say both positions are in the database but only the control value will be listed in the reports and exported. If duplicate points are found in the database and one is Control and the other of a lower classification and the Merge Duplicate points option is executed, the lower class will move to the Control position. If duplicate points are imported with the same classification (Control for example), the points will merge and the latest position will be used. If the tolerances under Project Settings/ Computational Settings/Point Tolerances are violated, the point will be red under the Project Explorer and it will be Flagged on the screen and listed under the Flags Pane. Both positions are in the database however and either one can be deleted.

2. There is another level to the point positions well, in the Trimble geodetic world there are Grid, Local (NAD83 for example) and Global (WGS84). Unless told otherwise TBC treats Local and Global as being the same (they are NOT). All types can be keyed in, but only once. However, multiple versions of the same type can be imported (but not in the case of DAT/T01 files, see below – RTK/DC files can however be imported multiple times).

3. DAT/T01 files have a particular status since they contain raw positions that have not yet been processed. In the case of a Static survey, points will be imported numerous times and MUST be named the same even though the files have somewhat different positions. In this case the latest (although temporary) position is used if the point does not have another position of a higher quality. However, this is ultimately moot since after processing and

adjustment a single position will be created with an Adjustment quality level.

Obviously, this data flow can get very complicated!

BOTTOM LINE – WHEN DEALING WITH MULTIPLE POSITION SOURCES (IMPORTED, KEYED IN, ADJUSTED, OR MERGED) BE SURE TO REVIEW THE POINT “HISTORY” UNDER THE PROJECT EXPLORER.

POINT SELECTION(s)

SELECT/Select Points and Advanced Select – basically the Select Points option is more rigid in that other than the software provided selections the user can only introduce limited filters (wildcards, partial strings, etc). Advanced Select allows for the user to specify a Data Type (Multiple Data Types, Points, Coordinates, etc) and then allows specific interrogation of details using criteria like Not Equal To Regular Expression, Not Equal, Equal and Equal to Regular Expression, with user input values. Note that the expressions used in TBC are international rather than Microsoft - see Select Using Advanced Criteria under Help for complete options.



